Target in Range

An Arduino Sonar Obstacle Detector Project

Written with the use of C and the AVR Assembly Language

By The Delta Group

Δ

Nick Kasten   -   Jesse Lew   -    Patrick Martinez

- Introduction

When deciding on which project we were going to pursue, our group decided to combine all of our ideas. We decided to use a sonar obstacle detection module, a tri-colored RGB LED, and a button connected to a buzzer.

Our biggest challenge was merging all of these different parts into the same program. We were able to tie everything together in C, which was our prototype version. We decided to use this first so that we could have an idea of how the program will work and what the requirements of our sonar function would be before starting on that piece of more complicated code.

After we completed our C version of the code and confirmed its functionality, we began putting our efforts towards removing the sonar function from the main file and attaching it as an external call to an AVR function.

To accomplish this, we converted the sonar function into a .lst and use it to make a .S using Professor Black's Arduino makefile. However, we were told not to do it that way, so we went back and rewrote it by hand. It ended up being much cleaner and significantly shorter (from 500 lines of Assembly down to 50!). It was easier not to have to worry about including any of the Arduino IDE's files, and to simply assign ports and pins ourselves.

2) The Microcontroller Platform

The platform we used was an Arduino Uno equivalent device, the Elegoo Uno. The Elegoo Uno that we obtained is a microcomputer functionally equivalent to the Arduino Uno available at a much lower cost. These may be obtained through many online supermarkets such as www.Amazon.com or www.ebay.com.

Fig 1. Elegoo Uno

The Elegoo Uno features an ATmega328 Microcontroller with an 8-bit processor and 16MHz clock speed. It also features a modifiable memory bank to load/run programs. The board has multiple I/O ports to send and receive signals. A Power port so it can run without the USB, and a USB port capable of providing power and writing data.

Fig. 2 Arduino Uno with port locations

3.a) The Test Device

The main test device in our project is an ultrasonic obstacle sensor, the HC-SR04.

Fig 3. HC-SR04 Sonar Obstacle Detector

This device has 4 pins:

- VCC – 5V, positive end of the power supply

- TRIG – Trigger Pin

- ECHO – Echo Pin

- GND – 0V, ground, negative end of the power supply

TRIG and ECHO were used to interface this module with our microcontroller, the Arduino. These are TTL (0 – 5V) input output pins.

3.b) How it works

Fig. 4 Simplified Diagram of How the Sonar Works

We need to:

- Provide a TRIGGER signal, at least 10μS High Level (5V pulse)

- The module transmits 8 40KHz ultrasonic bursts

- If there is an obstacle in-front of the module, it will reflect the ultrasonic burst. If the signal comes back, ECHO output of the sensor will be in HIGH state (5V) for the duration of time taken for sending and receiving the ultrasonic bursts. Pulse width ranges from about 150μS to 25mS. If no obstacle is detected, the echo pulse width will be about 38ms.

- We set a timer to measure that duration of time and can calculate the distance.

Fig. 5 How Sonar Detection Works

3.c) Other Gizmos

Our C program receives an integer value back from the Assembly Sonar function and we use that value to determine if a target is in range or not. We then display the proper LED light color.

Red = Out of range.

Yellow = Almost in range...

Green = Target in range!

Note: we used a RGB LED. So to create the different colors we wanted, we had to turn on different combinations. Specifically, to create Yellow, we had to turn on both Blue and Green at the same time.

When the target is finally in range, we enable a button to be pushed. The button is normally disabled and does nothing when a target isn't in range. The button sends a signal to a buzzer on the breadboard signifying that the target has been fired upon.

Target destroyed!

Development Tools

Generally, folks use the Arduino IDE to program their Arduino Device. We could not use the default as that would beat the point of this learning experiment. We used gVIM and sublime text as the main text editors and to examine our existing text. We extensively used gitHub to share and collaborate. We used Slack chat messaging app as a meeting place and to pass around ideas. And some of us used VMWare player for easier compilation and better integration when working as a team. We also used Fritzing to create a wiring diagram for our report.

Our Experiment

We decided pretty early in our project that we wanted to use the Sonar Obstacle Detector. We were planning to create our program in C and port out a small part of the program into assembly, like the LED light, but we were informed that this would be to simple and we decided to take on the challenge of programming the sonar device in C. Our project is sort of like a “rangefinder” missile test. The idea is that our system will detect if an object is at an optimum distance, then only allow it to FIRE if in range.

Fig. 6 Fritzing diagram set-up

The preceding diagram shows how our devices were connected to the microcontroller. It seems like a lot of wires, but it wasn’t too difficult to manage once we got a sense of where each one went. As mentioned before, we started by programming the entire system in C, then ported out the sonar as we wanted to set down our framework for the .S portion.

Conclusions

We were successful in our goal of creating a working Arduino sonar project that made use of both C code and Assembly. The device works as intended. It’s great to have a complete AVR sonar file that we can use in future Arduino projects. Group Delta Master Race!

Contributions

- Nick Kasten - Contributed to the planning stage, the hardware code, the entire software code and testing and troubleshooting efforts

- Jesse Lew - Contributed to the planning stage, extensive assembly research, this report and troubleshooting.

- Patrick Martinez  - Contributed to the planning stage, C and Assembly linking research, this report and troubleshooting.

Project Code

main.c

|  | #include <avr/io.h> |
| --- | --- |
|  | #include <util/delay.h> |
|  | /************************************************************************/ |
|  | /* Defining Constants */ |
|  | /************************************************************************/ |
|  | //--- PIN Assignments ---------------------------------------------------- |
|  | #define RED_PIN 3 |
|  | #define GREEN_PIN 4 |
|  | #define BLUE_PIN 5 |
|  | #define BUZZ_PIN 1 |
|  | //--- PIN Controls ------------------------------------------------------- |
|  | #define RED_ON (PORTB ¦= (1<<RED_PIN)) |
|  | #define RED_OFF (PORTB &= ~(1<<RED_PIN)) |
|  | #define GREEN_ON (PORTB ¦= (1<<GREEN_PIN)) |
|  | #define GREEN_OFF (PORTB &= ~(1<<GREEN_PIN)) |
|  | #define YELLOW_ON (PORTB ¦= (1<<RED_PIN) ¦ (1<<GREEN_PIN)) |
|  | #define YELLOW_OFF (PORTB &= (0x01)) |
|  | #define BUZZ_ON (PORTB ¦= (1<<BUZZ_PIN)) |
|  | #define BUZZ_OFF (PORTB &= ~(1<<BUZZ_PIN)) |
|  | //--- Device Setup ------------------------------------------------------- |
|  | #define OUTPUT_CONFIG (DDRB ¦= (1<<RED_PIN) ¦ (1<<GREEN_PIN) ¦ (1<<BUZZ_PIN)) |
|  | #define CPU_PRESCALE(n) (CLKPR = 0x80, CLKPR = (n)) |
|  | int buzzerState = 1; |
|  | /************************************************************************/ |
|  | /* Device function definitions */ |
|  | /************************************************************************/ |
|  | //--- Buzzer ------------------------------------------------------------- |
|  | void toggleBuzz(void) { |
|  | if (buzzerState == 1) { |
|  | BUZZ_OFF; |
|  | buzzerState = 0; |
|  | } else { |
|  | BUZZ_ON; |
|  | buzzerState = 1; |
|  | } |
|  | } |
|  | void buzzEnable(void) { |
|  | char ticks = 0; |
|  | int blastCount = 0; |
|  | while(blastCount <= 100) { |
|  | while((TIFR0 & 0x01) == 0) {} |
|  | TIFR0 = 1; |
|  | ticks++; |
|  | if (ticks == 20) { |
|  | ticks = 0; |
|  | toggleBuzz(); |
|  | } |
|  | blastCount++; |
|  | } |
|  | } |
|  | //--- LED ---------------------------------------------------------------- |
|  | void LEDColor(int distance) { |
|  | YELLOW_OFF; |
|  | if (distance >= 4) { |
|  | RED_ON; |
|  | } |
|  | else if (distance >= 2) { |
|  | YELLOW_ON; |
|  | } |
|  | else { |
|  | GREEN_ON; |
|  | buzzEnable(); |
|  | } |
|  | } |
|  | /************************************************************************/ |
|  | /* MAIN function */ |
|  | /************************************************************************/ |
|  | int main(void) { |
|  | //--- Setup ---------------------------------------------------------- |
|  | int distance; |
|  | OUTPUT_CONFIG; |
|  | // Set Timer/Counter to proper speed for buzzer |
|  | TIMSK0 = 0; |
|  | TCCR0B = 2; |
|  | //--- Main Loop ------------------------------------------------------ |
|  | while(1){ |
|  | distance = sonar(); |
|  | LEDColor(distance); |
|  | } |
|  | return 0; |
|  | } |

Sonar.S

;-----------------------------------------------;              
;     ASM code for the HC-SRO4 Sonar device     ;
;         Created by: The Delta Group           ; 
;-----------------------------------------------;

#include "config.inc"

	.section        .text
	.global		sonar

;-----------------------------------------------;
;               SONAR FUNCTION                  ;
;-----------------------------------------------;
sonar:
	
        sbi             _DDRD, TRIG_PIN         ; sets pin 2's direction to output => Trig Pin
        cbi             _DDRD, ECHO_PIN         ; sets pin 4's direction to input  <= Echo Pin
        cbi             _PORTD,	TRIG_PIN        ; sets the output to LOW
send:						
        sbi             _PORTD, TRIG_PIN        ; set Trig Pin to HIGH to send a burst of 8 pings
        call            tenMicroDelay           ; HIGH signal must continue for 10 microseconds
        cbi             _PORTD,	TRIG_PIN        ; set the output to LOW after pings are sent
receive:
        clr             r21                     ; clear counter value
waiting:
        call            hundredCMDelay          ; wait long enough for the sound to travel 100 cm (each way)
        inc             r21                     ; increment counter
        sbic            _PIND, ECHO_PIN		; break out of loop if response received
        rjmp            waiting                 ; otherwise keep waiting for the response
        clr		r25			; zero out high bits of return register
        mov		r24, r21                ; set lower bits to the value of counter
        call            tenMicroDelay
        ret					; return to calling function

;-----------------------------------------------;
;               DELAY FUNCTIONS                 ;
;-----------------------------------------------;
tenMicroDelay:
        ldi             r18, 2                  ; set outer loop to 2
1:
        ldi             r19, 80                 ; set inner loop to 80 (160 cycles total)
2:
        dec             r19                     ; decrement inner loop
        brne            2b                      ; keep looping until (r19 == 0)
        dec             r18                     ; decrement outer loop
        brne            1b                      ; keep looping until (r18 == 0)
        ret                                     ; return after 10 microseconds
hundredCMDelay:
        ldi             r18, 100                ; set outer loop to 100 (centimeters)
5:
        ldi             r19, 58                 ; set middle loop to 58 (microseconds per centimeter)
6:      
        ldi             r20, 16                 ; set inner loop to 16 (cycles per microsecond)
7:
        dec             r20                     ; decrement inner loop
        brne            7b                      ; keep looping until r20 == 0
        dec             r19                     ; decrement middle loop
        brne            6b                      ; keep looping until r19 == 0
        dec             r18                     ; decrement outer loop
        brne            5b                      ; keep looping until r18 == 0
        ret                                     ; return after a delay equal to 100 centimeters